Border Gateway Protocol

Purpose: 
BGP is a comonly used routing protocol through out the internet and it is important to understand the common applications and configurations that it often operates under.

Background

Border Gateway protocol allows for internetwork or routing protocol connectivity. It does this by establishing neighbors and having a way to translate the priorities of one routing protocol to another one. There a two types of BGP, iBGP which acts as a routing protocol for devices, but eBGP allows for communication of routes between protocols. BGP will also allow for fastest path to be accounted for through networks as well furthermore it also keeps local path preference as well. While there have been some issues with the ability to propagate incorrect bgp routes to the point that a website will become inaccessible regardless of where you start from on the internet (something that was an issue on our local network due to some improperly entered IP addresses). This is known as blackholing which prevents the packets from being routed to the proper location instead of being sent to a location at which the address doesn’t exist. Some issues with BGP have been fixed with frameworks like RPKI.

Luckily most ISPs use these safety frameworks which prevent unnecessary internet outages. BGP is also good because it can be configured to provide extra information as well as optimizing so that it doesn’t require that much network bandwidth. BGP also has rout advertisements and updates that allow it to function as an internal routing protocol as well (iBGP) which uses TCP. It also solves issues like routing policy a need for inter-domain routing at a large scale as well as preventing loops once different Autonomous systems are connected. In the past with other routing protocols connecting large networks would have required a large number of calculations by spanning tree or manually preventing loops with shutdown commands. Furthermore only best routes are provided to other devices running BGP.

The Other protocols we used was OSPF, ISIS, and EIGRP. These protocols provide unique advantages with EIGRP is the ease in which it can be configured while also providing routing and most basic requirements for routing protocols. OSPF is often used as it is able to have unique areas and so create smaller routing tables within them (although this still has the backbone area containing all of the routes). This allows for faster less necessary updates per topology change while keeping connectivity. It also is able to scale quite well up to quite high number routers. Both of these protocols were relatively easy to set up and can be scaled to larger sizes if required by a company and they have their own distinct use cases with EIGRP being useful for small businesses and SOHO environments while OSPF is more capable of handling the number of devices on a businesses, school, or other large network and is widely used and where there may be some topology changes It is worth noting that if you ever want to add IPv6 routing to your network you should start with OSPv3 as if you only do OSPF you will need to reconfigure both IPv4 and IPv6 addresses.. Similarly to OSPF ISIS is also quite scalable although from personal experience it is significantly more unintuitive to learn how to set up. ISIS is less often used and is faster at propagating routing information around a network than OSPF and used by some service providers as it is able to maintain a lot of routes.

Configuration:

hostname R1

boot-start-marker

boot-end-marker

vrf definition Mgmt-intf

address-family ipv4

exit-address-family

address-family ipv6

exit-address-family

no aaa new-model

login on-success log

subscriber templating

vtp domain cisco

vtp mode transparent

ipv6 unicast-routing

multilink bundle-name authenticated

crypto pki trustpoint TP-self-signed-414644419

enrollment selfsigned

subject-name cn=IOS-Self-Signed-Certificate-414644419

revocation-check none

rsakeypair TP-self-signed-414644419

crypto pki certificate chain TP-self-signed-414644419

certificate self-signed 01

quit

license udi pid ISR4321/K9 sn FDO21080CTW

no license smart enable

diagnostic bootup level minimal

spanning-tree extend system-id

redundancy

mode none

interface Loopback0

ip address 192.168.0.1 255.255.255.252

ipv6 address 2001:DB8:ACAD::F1/64

ospfv3 1 ipv4 area 0

ospfv3 1 ipv6 area 0

interface GigabitEthernet0/0/0

ip address 10.10.1.1 255.255.255.0

negotiation auto

ipv6 address 2001:DB8:ACAD:1::1/64

ospfv3 1 ipv6 area 0

ospfv3 1 ipv4 area 0

interface GigabitEthernet0/0/1

no ip address

negotiation auto

interface Serial0/1/0

no ip address

shutdown

interface Serial0/1/1

no ip address

shutdown

interface Service-Engine0/2/0

interface GigabitEthernet0

vrf forwarding Mgmt-intf

no ip address

shutdown

negotiation auto

router ospfv3 1

router-id 2.1.1.1

address-family ipv4 unicast

exit-address-family

address-family ipv6 unicast

exit-address-family

router ospf 1

router-id 1.1.1.1

network 10.10.1.0 0.0.0.255 area 0

network 192.168.0.1 0.0.0.0 area 0

ip forward-protocol nd

ip http server

ip http authentication local

ip http secure-server

ip tftp source-interface GigabitEthernet0

control-plane

line con 0

transport input none

stopbits 1

line aux 0

stopbits 1

line vty 0 4

login

End

hostname R2

boot-start-marker

boot-end-marker

vrf definition Mgmt-intf

address-family ipv4

exit-address-family

address-family ipv6

exit-address-family

no aaa new-model

ipv6 unicast-routing

subscriber templating

vtp domain cisco

vtp mode transparent

multilink bundle-name authenticated

license udi pid ISR4321/K9 sn FDO214328EH

spanning-tree extend system-id

redundancy

mode none

vlan internal allocation policy ascending

vlan 10,20

interface Loopback0

ip address 192.168.2.1 255.255.255.255

ipv6 address 2001:DB8:ACAD:F::1/64

ospfv3 1 ipv4 area 0

ospfv3 1 ipv6 area 0

interface GigabitEthernet0/0/0

ip address 10.10.2.1 255.255.255.0

negotiation auto

ipv6 address 2001:DB8:ACAD:2::2/64

ospfv3 1 ipv4 area 0

interface GigabitEthernet0/0/1

ip address 10.10.1.2 255.255.255.0

ip ospf 1 area 0

negotiation auto

ipv6 address 2001:DB8:ACAD:1::2/64

ipv6 ospf 1 area 0

interface Serial0/1/0

no ip address

shutdown

interface Serial0/1/1

no ip address

shutdown

interface Service-Engine0/2/0

no ip address

shutdown

interface GigabitEthernet0

vrf forwarding Mgmt-intf

no ip address

shutdown

negotiation auto

interface Vlan1

no ip address

shutdown

router ospfv3 1

router-id 1.2.2.2

address-family ipv4 unicast

redistribute connected

redistribute bgp 2 metric 10

exit-address-family

address-family ipv6 unicast

passive-interface default

no passive-interface GigabitEthernet0/0/1

redistribute connected

redistribute bgp 2 metric 10

exit-address-family

router ospf 1

router-id 1.2.2.2

redistribute bgp 2 metric 10 subnets

network 10.10.1.0 0.0.0.255 area 0

network 192.168.2.0 0.0.0.255 area 0

router bgp 2

bgp router-id 2.2.2.2

bgp log-neighbor-changes

redistribute connected

redistribute ospfv3 1 metric 10

redistribute ospf 1 metric 10

neighbor 10.10.2.3 remote-as 3

ip forward-protocol nd

no ip http server

no ip http secure-server

ip tftp source-interface GigabitEthernet0

control-plane

line con 0

stopbits 1

line aux 0

stopbits 1

line vty 0 4

login

End

hostname R3

boot-start-marker

boot-end-marker

vrf definition Mgmt-intf

address-family ipv4

exit-address-family

address-family ipv6

exit-address-family

no logging console

no aaa new-model

no ip domain lookup

ipv6 unicast-routing

subscriber templating

vtp domain cisco

vtp mode transparent

multilink bundle-name authenticated

license udi pid ISR4321/K9 sn FDO214414TX

spanning-tree extend system-id

redundancy

mode none

vlan internal allocation policy ascending

vlan 10,20

interface Loopback0

ip address 192.168.3.1 255.255.255.255

ip router isis Trump

interface GigabitEthernet0/0/0

ip address 10.10.3.1 255.255.255.0

ip router isis Trump

negotiation auto

ipv6 address 2001:DB8:ACAD:3::3/64

ipv6 router isis Trump

interface GigabitEthernet0/0/1

ip address 10.10.2.3 255.255.255.0

negotiation auto

ipv6 address 2001:DB8:2::3/64

interface Serial0/1/0

no ip address

Shutdown

interface Serial0/1/1

no ip address

shutdown

interface GigabitEthernet0/2/0

no ip address

shutdown

negotiation auto

interface GigabitEthernet0/2/1

no ip address

shutdown

negotiation auto

interface GigabitEthernet0

vrf forwarding Mgmt-intf

no ip address

shutdown

negotiation auto

interface Vlan1

no ip address

shutdown

router isis Trump

net 49.0001.1010.1010.3333.00

redistribute connected

redistribute bgp 3 metric 10 level-1

router bgp 3

bgp router-id 3.3.3.3

bgp log-neighbor-changes

redistribute connected

redistribute isis Trump level-1-2 metric 10

neighbor 10.10.2.1 remote-as 2

ip forward-protocol nd

no ip http server

no ip http secure-server

ip tftp source-interface GigabitEthernet0

control-plane

line con 0

stopbits 1

line aux 0

stopbits 1

line vty 0 4

login

End

hostname R4

boot-start-marker

boot-end-marker

vrf definition Mgmt-intf

address-family ipv4

exit-address-family

address-family ipv6

exit-address-family

no aaa new-model

login on-success log

subscriber templating

vtp domain cisco

vtp mode transparent

ipv6 unicast-routing

multilink bundle-name authenticated

crypto pki trustpoint TP-self-signed-1275915594

enrollment selfsigned

subject-name cn=IOS-Self-Signed-Certificate-1275915594

revocation-check none

rsakeypair TP-self-signed-1275915594

crypto pki certificate chain TP-self-signed-1275915594

certificate self-signed 01

quit

license udi pid ISR4321/K9 sn FDO214811ZM

no license smart enable

diagnostic bootup level minimal

spanning-tree extend system-id

redundancy

mode none

interface Loopback0

ip address 10.2.2.2 255.255.255.255

ip router isis Trump

interface GigabitEthernet0/0/0

ip address 10.10.10.2 255.255.255.0

negotiation auto

ipv6 address 2001:DB8:ACAD:4::4/64

interface GigabitEthernet0/0/1

ip address 10.10.3.2 255.255.255.0

ip router isis Trump

negotiation auto

ipv6 address 2001:DB8:ACAD:3::4/64

ipv6 router isis Trump

interface Serial0/1/0

no ip address

shutdown

interface Serial0/1/1

no ip address

shutdown

interface GigabitEthernet0/2/0

no ip address

shutdown

negotiation auto

interface GigabitEthernet0/2/1

no ip address

shutdown

negotiation auto

interface GigabitEthernet0

vrf forwarding Mgmt-intf

no ip address

shutdown

negotiation auto

router isis Trump

net 49.0001.1010.1010.4444.00

metric-style narrow

redistribute connected

redistribute bgp 400 metric 10 level-1

router bgp 400

bgp router-id 4.4.4.4

bgp log-neighbor-changes

redistribute connected

redistribute isis Trump level-1-2 metric 10

neighbor 10.1.1.1 remote-as 500

neighbor 10.1.1.1 ebgp-multihop 2

neighbor 10.1.1.1 update-source Loopback0

neighbor 10.10.10.3 remote-as 500

ip forward-protocol nd

ip http server

ip http authentication local

ip http secure-server

ip tftp source-interface GigabitEthernet0

control-plane

line con 0

transport input none

stopbits 1

line aux 0

stopbits 1

line vty 0 4

login

End

hostname R5

boot-start-marker

boot system flash bootflash:isr4300-universalk9.16.09.08.SPA.bin

boot-end-marker

vrf definition Mgmt-intf

address-family ipv4

exit-address-family

address-family ipv6

exit-address-family

no aaa new-model

ip dhcp pool webuidhcp

login on-success log

subscriber templating

vtp domain cisco

vtp mode transparent

multilink bundle-name authenticated

crypto pki trustpoint TP-self-signed-2219300048

enrollment selfsigned

subject-name cn=IOS-Self-Signed-Certificate-2219300048

revocation-check none

rsakeypair TP-self-signed-2219300048

crypto pki certificate chain TP-self-signed-2219300048

certificate self-signed 01

quit

license udi pid ISR4321/K9 sn FLM240607T3

no license smart enable

diagnostic bootup level minimal

spanning-tree extend system-id

redundancy

mode none

interface Loopback0

ip address 192.168.5.1 255.255.255.255

interface GigabitEthernet0/0/0

ip address 10.10.5.1 255.255.255.0

negotiation auto

ipv6 address 2001:DB8:ACAD:5::5/64

interface GigabitEthernet0/0/1

ip address 10.10.10.3 255.255.255.0

negotiation auto

ipv6 address 2001:DB8:ACAD:4::5/64

interface GigabitEthernet0

vrf forwarding Mgmt-intf

no ip address

shutdown

negotiation auto

router eigrp 100

network 10.0.0.0

network 10.10.5.0 0.0.0.255

network 192.168.5.0

redistribute connected

redistribute bgp 500 metric 10000 100 255 1 1500

passive-interface default

no passive-interface GigabitEthernet0/0/0

eigrp router-id 5.5.5.5

router bgp 500

bgp router-id 5.5.5.5

bgp log-neighbor-changes

redistribute connected

redistribute eigrp 100

neighbor 10.10.10.2 remote-as 400

ip forward-protocol nd

ip http server

ip http authentication local

ip http secure-server

ip tftp source-interface GigabitEthernet0

control-plane

line con 0

transport input none

stopbits 1

line aux 0

stopbits 1

line vty 0 4

login

End

hostname R6

boot-start-marker

boot-end-marker

vrf definition Mgmt-intf

address-family ipv4

exit-address-family

address-family ipv6

exit-address-family

no aaa new-model

subscriber templating

vtp domain cisco

vtp mode transparent

multilink bundle-name authenticated

license udi pid ISR4321/K9 sn FDO214913GF

spanning-tree extend system-id

redundancy

mode none

vlan internal allocation policy ascending

vlan 10,20

interface Loopback0

ip address 192.168.6.1 255.255.255.255

interface GigabitEthernet0/0/0

no ip address

negotiation auto

interface GigabitEthernet0/0/1

ip address 10.10.5.2 255.255.255.0

negotiation auto

ipv6 address 2001:DB8:ACAD:5::6/64

interface Serial0/1/0

no ip address

shutdown

interface Serial0/1/1

no ip address

shutdown

interface GigabitEthernet0/2/0

no ip address

shutdown

negotiation auto

interface GigabitEthernet0/2/1

no ip address

shutdown

negotiation auto

interface GigabitEthernet0

vrf forwarding Mgmt-intf

no ip address

shutdown

negotiation auto

interface Vlan1

no ip address

shutdown

router eigrp 100

network 10.0.0.0

network 192.168.6.0

redistribute connected

passive-interface default

no passive-interface GigabitEthernet0/0/1

ip forward-protocol nd

no ip http server

no ip http secure-server

ip tftp source-interface GigabitEthernet0

control-plane

line con 0

stopbits 1

line aux 0

stopbits 1

line vty 0 4

login

End

Network Diagram

Commands Used

Network (IP address of interface) route-map (name of route map) in

This allows the protocol to know what networks are using it for routing

This also allows for more specific information like cost or weight to be established here instead of individually per IP address

Redistribute connected

This shares the known networks from a router to the next connected devices
Router BGP 100

This allowed for connectivity between other routing protocols (needed some more commands for this like with metric)

Router ISIS Trump

This allows for the usage of ISIS with a few other commands needed

Router OSPF area 0

This along with the proper configuring of an interface can allow for OSPF to be run

Problems experienced

It was often problematic to find specific metrics as i was unable to tell for which specific routing protocol and which side of the protocol needed the command as most examples were just one routing protocol to BGP and not Routing protocol-BGP then next router going from BGP to another routing protocol. This caused some of my configurations to work only one way and the Usage of ISIS in the center of this was not helpful in my attempt to make this a simplistic process.  Since we didn’t start with route maps in the beginning of our configuration, the implementation of them was sketchy as I wasn’t always sure if my route map was allowing the routing to work of if it was the older parts of the config. Furthermore in the process of switching from only network commands to network commands and route maps I miss typed 2 IP addresses which caused a complete break in connectivity which took a couple of days of pure debugging to fix. This also extended to more problems within the network as rereading numbers can be very tedious and unenjoyable, thus decreasing my motivation and focus when it is happening within my config. This was only remedied by receiving help from other members of my class after asking one of them to help me debug my configuration which we reviewed the entirety of the configuration and (due to the aforementioned tedium of reviewing IP addresses and metrics) looked at metrics last which is where the issue was eventually uncovered and quicky fixed.

Other issues I encountered was the formatting for ISIS area IDs as when looking online it looked quite complex and hard to manage. We were able to eventually work out what needed to be done but the terminology and resources we found were unhelpful and problematic for our understanding of the protocol making it seem excessively complex to set up and work with. The unique weight number was also problematic and it explains much of the struggle and decision making that was made by my fellow classmates as many of them that started with a configuration similar to ours quickly switched away from it with a different protocol in the center of the network.

Conclusion:

BGP is an important tool for the internet and can be implemented on many personal networks as well as it can allow for the division of your network depending on the needs of your various users. It can be verry complex with extensive information about these routes and the transmitting of them. This protocol can be verry beneficial and has taught me the necessity of slowing down, being patient and consistent rather than quicky skipping over some parts and struggling to debug later.